A PROGRAM TO TRANSLATE PARALLAX SX-KEY LIST FILES INTO A FORM WHICH

THE LOREN BLANEY SXSIM SIMULATOR UNDERSTANDS.

The translation program  (trans.exe ) was written in Quickbasic. It takes the "code.lst" file generated by the sxkey assembler and creates a new text file “nucode.lst”, which is then used as the input to SxSim. This allows all the instructions used by the Parallax assembler to be simulated on SxSim.

In order to use the existing executable version of the translator you should do the following :-

( The following instructions assume that you are using Windows 95 )

- Create a new folder “c:\scenix\sxkey_e

- Put the files sxsim.exe, sxkey_084.exe, trans.exe, sst.bat, L50.com, and L25.bat into this folder.

- Create a new desktop shortcut (sxsim) with the command line “c:\scenix\sxkey_e\sst.bat “, a working directory of “c:\scenix\sxkey_e “, and set up to run full screen.

The program is now ready to use, in the following way :-

- Run “sxkey_084.exe “, and open your source file (The translator works best if you separate each field in the source code with tabs e.g label<tab>instruction<tab>operand<tab>;comment ).

- From the “Run” menu select “Listing”. The assembler now creates a file called “code.lst”. This is always the same filename regardless of the source filename.

- Double click the “sxsim” shortcut which you created. This will take the “code.lst” file , create “nucode.lst” and run Sxsim using this file.

Notes :-

- The translation program doesn’t currently handle the sx fuse settings so you need to input these manually. I will try to alter this when possible.

- The folder name can be different only if you change the first two lines in the “trans.bat” file and recompile the executable version.

The Loren Blaney SxSim simulator is a superb development tool. By combining this with the translation program and Sxkey , you create an integrated development environment.

Geoff Jackson

Jackson-Woodburn Controls.

November ’98

GEOFFJACKSON@MSN.COM